# WINDOW WATER PENETRATION FIELD TEST REPORT

PROJECT NAME:

{{ project_name }}

LOCATION:

{{ project.address }}

REPORT TO:

{{ project_client }}

| PROJECT NUMBER: | {{ project_id }} |
| --- | --- |
| TEST DATE: | {{ visit.date }} |
| REPORT DATE: | 7/13/2026 |

## SCOPE:

BEE Engineers was contracted by {{ project.client }} for the purpose of performing on-site water penetration testing on vinyl window installations.

## METHOD:

ASTM E1105-15, Standard Test Method for Field Determination of Water Penetration of Installed Exterior Windows, Skylights, Doors, and Curtain Walls, by Uniform or Cyclic Static Air Pressure Difference (2023)

## SPECIFICATIONS:

Test pressures provided by BEE Engineers

## PRODUCT TYPE(S):

{{ summary.product_types_list_text }}

## APPARATUS:

Chamber: The test chamber was constructed on-site using wooden framing and covered with a transparent vinyl exterior. The chamber’s perimeter was attached and sealed to the framing studs located on the interior side of the wall.

Pressure Differential: The interior chamber was depressurized using a Retrotec 200 Series Fan. A reference tube was placed inside the chamber through the vinyl skin and attached to one sensor input on A Dwyer 477AV Series Manometer  while the other input monitored the ambient pressure to measure the negative pressure differential across the assembly.

Water Application: A water spray rack consisting of a 1” PVC tube grid with Spraying Systems Co. - 1/4BX

5 Whirljet Brass nozzles mounted on the exterior at a distance of 15” ± 2” to deliver water uniformly against the exterior surface of the test specimen.

## PROCEDURE:

Products were tested in conformance with ASTM E1105 procedures:

Procedure A - Uniform static air pressure difference.

Consists of maintaining the specified static air pressure difference across the test specimen with the specified rate of water spray for 15 minutes.

Procedure B - Cyclic static air pressure difference.

Water was sprayed on to the exterior face of the assembly at the required rate of 5 gph/sqft. A negative pressure differential was applied within 15 secs and was maintained for 5 min. Water spray was maintained while the pressure differential was reduced to zero and held for 1 minute, this process was repeated 4 times for a total test time of 24 minutes.

Refer to the individual test parameters for the procedure used. Exterior doors were open throughout the duration of the test to maintain balanced static pressure between exterior and interior locations. The interior face of the test area was observed for points of water leakage throughout testing.

## Sampling Procedure:

Test samples are chosen to represent typical conditions and details found on the project. Selection is based on the following considerations:

- Configuration of Details: Include assemblies that reflect common design and installation conditions.

- Exposure: Prioritize specimens that are installed in their final, in-situ position and subject to actual environmental conditions.

- Construction Schedule: Align sample selection with the project timeline to minimize disruption and ensure readiness for testing.

- Installation Progress: Consider the percentage of the glazing system completed to ensure representative coverage.

- Accessibility: Select locations that allow safe and practical setup of test apparatus and observation.

## PERFORMANCE CRITERIA:

ASTM E1105-15 3.2.3: Water penetration shall be defined as penetration of a plane parallel to the glazing (the vertical plane) intersecting the innermost projection of the test specimen, not including interior trim and hardware, under the specified pressure difference across the specimen. Any such water penetration shall constitute failure of the water penetration resistance test. It shall also constitute failure if water penetrates through the perimeter frame of the test specimen. Water contained within drained flashing, gutters, and sills shall not be considered a failure.

## Attendance:

The following representatives were present during all or part of the tests:

## TECHNICIANS:

| Name: | Company: | Role: |
| --- | --- | --- |
| {{ personnel.lead_technician }} | BEE Engineers | Lead Technician on-site |
| {{ personnel.technician_2 }} | BEE Engineers | Technician |

## WITNESSES:

The following representatives were present during all or part of the tests:

| Name: | Company: | Role: |
| --- | --- | --- |
| N{{ witnesses.witness_name_1 }} | {{ witnesses.witness_company_1 }} | {{ witnesses.witness_role_1 }} |
| N{{ witnesses.witness_name_2 }} | {{ witnesses.witness_company_2 }} | {{ witnesses.witness_role_2 }} |

## COMPLIANCE STATEMENT:

Results were obtained through tested values and acquired using methods designated in ASTM E1105. All

equipment used to perform testing was in current calibration per the standard.

## GENERAL NOTES:

Pressure differential and duration of application was determined by project specification.

Tested samples were inclusive of adjacent construction materials and installation details.

All locations referenced are described as viewed from the interior unless otherwise noted.

A detailed visual examination of each test sample and adjacent construction was conducted prior to testing. No deficiencies were noted.

## TEST SUMMARY:

{{ summary.product_types_sentence }}.

The failed location will need to have a remediation protocol approved by the design team prior to implementing a fix. Upon completion of the approved remediation, failed locations shall be retested to achieve a passing result. An additional three locations per failure of the same type and configuration will need to be tested to ensure the problem is not systemic. For Individual tests details see Appendix A.

| PREPARED BY: | REVIEWED AND APPROVED BY: |
| --- | --- |
| {{ personnel.lead_technician }} Lead Technician On-Site | {{ manager.pm_name_with_info }} {{ manager.pm_title }} |
| ATTACHED: Appendix A – Test Results Project Summary Appendix B – Locations & Drawings |  |

## DISCLAIMER: Please note that there is no guarantee expressed, written, or implied by this report. The data compiled herein is subject to change in case of new or additional data discovery. The information contained in this report is based on information available at the time that this report was prepared. Concerns do not represent a total listing of all locations with issues, nor do they imply all similar locations will have issues. If additional information is discovered, it should be forwarded to our office for further review and comment. We reserve the right to amend and/or modify this report if new and/or significant data becomes available that impacts the situation and parameters of this investigation. Our services were performed using the degree of skill normally exercised by practicing investigation individuals in this area and similar locales. No other warranty is either expressed or implied.

# APPENDIX A

# TEST RESULTS

{% for s in samples %} SAMPLE {{ s.sample_id }}

— FIELD WATER PENETRATION TEST

(ASTM E1105)

| Location and Test History |  |  |  |
| --- | --- | --- | --- |
| Elevation: | {{ s.elevation }} | Test Date: | {{ s.test_date }} |
| Floor Level: | {{ s.floorLevel }} | Test Time: | {{ s.test_time }} |
| Unit Number: | {{ s.unitNumber }} | Test Phase: | {{ s.test_phase }} |
| Description: | {{ s.locationDescription }} |  |  |
| Sample Details: |  | Test PARAMETERS: |  |
| Manufacturer: | {{ s.manufacturer }} | Procedure: | {{ s.procedure }} |
| Series/Model: | {{ s.series_model }} | Pressure Differential: | {{ s.pressure_psf }}psf ({{ s.pressure_inwc }}inWC) |
| Product Type: | {{ s.system_type }} | Atmospheric Temp: | {{ s.atmosphericTemp_f}}°F |
| Product Size: | {{ s.width_in }}” x {{ s.height_in}}” | Barometric Pressure: | {{ s.barometricPressure_inhg}}inHg |
| Configuration: | {{ s.configuration }} | Wind Speed (dir): | {{ s. windSpeed_mph }} mph ({{ s.windDirection }}) |

| TEST RESULT: |  |  |  |
| --- | --- | --- | --- |
| {{ s.result }}. {% if s.result == "FAIL" %} Cycle {{ s.failure_cycle }} ¦ Time {{ s.failure_time }} ¦ {{ s.failure_mode }} ¦ {{ s.failure_location }} {% else %}The test sample met performance requirements at the specified test pressure. {% endif %} {% endfor %} |  |  |  |

| {{ s.photoExterior }} | {{ s.photoExterior }} |
| --- | --- |
| Exterior Spray Rack | Interior Chamber |

# PROJECT SUMMARY

| SITE VISIT | DATE | DESCRIPTION |
| --- | --- | --- |
| 1 | 2026/01/06 | SCOPE: : {{ summary.product_types_list_text }} RESULTS: Pass: {{ summary.pass_count }} Fail: {{ summary.fail_count }} |

# APPENDIX B

# LOCATIONS & DRAWINGS

# PRODUCT DRAWINGS

# INSTALLATION DETAILS

# TEST LOCATIONS